Do at your own risk; this mod was completed on a 98.5 A4.

This mod gives a very stock red glow at your feet; it is not distracting at all in my mind.

Supplies: red 12V lights (Radio Shack Part #272-332), hook-up wire, fuse tap and female quick connect, ground loop and Strippers and of course electric tape.

This took about 15 to 30 mins to complete, the pickier you are about placement the longer it takes.

First decide how you want to power the lights, after some testing I found that my fog light fuse only had power when my lights are on. So determine which fuse is the fog if you want that feature. Also decide where you want the bulbs placed. Make sure the bulb is not directly visible to the driver.

Now run the wire along the back of the foot boxes and at each point you want to add a light just splice the cord in that area. By splicing I mean strip the wire without cutting it in half and connect the bulbs to it make sure to tape the wire and that bare wire is not touching each other. I put two bulbs in each foot box. At the end of the wire you can either have the last bulb connected there or cap them so that they will never touch. It is also possible to wire these in a series wiring diagram, but I didn't think of that till after.

After wiring and bulbs are connected and placed. Use the loop to ground the one wire and the fuse tap to give it power. Turn your lights on and see your work.

This write up was done by Ron Overs aka BigRonsAudi