Lighted Foot Box Instructions

This is a write up for lights powered by a laminated switch. You will need:

(given are some of the part numbers from products purchased at radio shack)

hook-up wire (278-567 25 feet of 18 Gauge wire, Red/black)

fuse tapper (270-1204)

also a  female connector for the fuse tapper

Ground Loop (64-3032 Assorted ring crimp on style)

Wire Strippers (whatever kind you want)

12V Switch (Any switch you want, If you go with one without a light in it you do not need to run a ground to it)

12V Lights (I used neons, pick what you would like)

Zip ties for securing

inline fuses if you like (safer, I went without)

and some small hands (something I don’t have so it took me longer)

Of course this is a do at your own risk, and may be illegal in some area, yada yada yada

- Decide where you are going to place the switch. I used the cigarette tray so I will refer to that as my location. I drilled a hole in the bottom of the tray and used the switch mounting bracket in the tray.

- Second run the wire you will need to run enough wire to go from the fuse box, under the carpet behind the pedals and then to your switch, make sure to leave a good amount extra.

- Feed the wire to the area where the switch will be place and leave an extra 6 inchs to work with. To get it into the cigarette tray I took the tray out and used a coat hanger to push down and then fished the wire up.

- Connect the power cord to the switch power source, the ground to the ground, and then use a single cord to wire from the lead on the switch back to the neons as the power.

- Run the lead power cord to the back of the counsel. Here you will connect the neons power wires to this. For the ground you can either tap the ground going to the switch or run a separate ground to the screw. I just spliced the ground and attached there. Note: If your neons come with cigarette adaptors cut them off and figure out which cord is power and ground.

- To mount the neon I used 4 large zip ties and 4 small ones. I used the large ones to mount to parts of the car in the foot box and then the small ones to connect the neons to the large ones. I found the 2 sided tape did not work for me, but you may have better luck if you want to try.

- There are many ways to get the power supply for the neons.(i.e. off the battery, fuse box or splicing the wire) I used the fuse box and used the cigarette lighter fuse. It has constant power so the neons can be on with the car off. Take the fuse out and put the tap on the fuse and put it back into the fuse holder. Crimp the power wire to the female quick connect and then connect the female quick connect to the fuse tap.  Find a screw around the fuse box area (I found one above it the fuse box) and crimp the ground to the loop and to ground it to the screw.

- Turn on the switch and see how you did.